# Gesture-Controlled Gazebo Car Worklog

This document explains how the SolidWorks car model, ROS 2 Foxy package, Gazebo simulation, and Windows hand gesture recognizer were connected into a working gesture-controlled car.

The final result is:

- The car model spawns in Gazebo.

- A Windows webcam gesture recognizer classifies hand gestures.

- The recognizer sends UDP JSON commands to WSL.

- A ROS 2 node in WSL receives those commands.

- The ROS 2 node directly controls the Gazebo model using Gazebo entity state services.

## Final Architecture

```text

Windows webcam

|

v

MediaPipe + trained gesture model

|

v

C:\Users\Yoav\Desktop\Personal Projects\hand-gestured-car\src\live_recognize.py

|

| UDP JSON on port 4210

v

WSL Ubuntu-20.04 / ros_joe

|

v

/home/ros_joe/HandGestureDrone_ws/src/car_assembly_2nd/scripts/gesture_gazebo_driver

|

| /get_entity_state and /set_entity_state

v

Gazebo model: car_assembly_2nd

```

V1 intentionally uses direct Gazebo model velocity control. It does not yet use wheel physics, a differential drive plugin, or ROS 2 control.

## Important Project Paths

Windows gesture project:

```text

C:\Users\Yoav\Desktop\Personal Projects\hand-gestured-car

```

Main gesture recognizer:

```text

C:\Users\Yoav\Desktop\Personal Projects\hand-gestured-car\src\live_recognize.py

```

Gesture model files:

```text

C:\Users\Yoav\Desktop\Personal Projects\hand-gestured-car\models\gesture_rf.joblib

C:\Users\Yoav\Desktop\Personal Projects\hand-gestured-car\models\labels.txt

```

ROS 2 workspace:

```text

/home/ros_joe/HandGestureDrone_ws

```

Active ROS 2 car package:

```text

/home/ros_joe/HandGestureDrone_ws/src/car_assembly_2nd

```

Active URDF:

```text

/home/ros_joe/HandGestureDrone_ws/src/car_assembly_2nd/urdf/car_assembly_2nd.urdf

```

Gazebo gesture driver:

```text

/home/ros_joe/HandGestureDrone_ws/src/car_assembly_2nd/scripts/gesture_gazebo_driver

```

Custom Gazebo world:

```text

/home/ros_joe/HandGestureDrone_ws/src/car_assembly_2nd/worlds/gesture_empty.world

```

## Gesture Recognition Side

The Windows project uses:

- OpenCV for webcam capture.

- MediaPipe Hands for hand landmarks.

- A trained model loaded from `models/gesture_rf.joblib`.

- Gesture labels from `models/labels.txt`.

The current gesture labels are:

```text

STOP_IDLE

SWIPE_LEFT

SWIPE_RIGHT

PUSH

PULL

CIRCLE_CW

CIRCLE_CCW

```

The recognizer uses a sliding time window of landmark motion and classifies the movement. It smooths predictions using:

- `STREAK_REQUIRED = 3`

- `CONF_THRESH = 0.70`

- `COOLDOWN_SECONDS = 0.6`

## Current Gesture Controls

The current control mapping is:

```text

CIRCLE_CW    -> arm

CIRCLE_CCW   -> arm

STOP_IDLE    -> disarm and STOP

PUSH         -> FORWARD

PULL         -> BACKWARD

SWIPE_LEFT   -> LEFT

SWIPE_RIGHT  -> RIGHT

```

Movement commands only work while armed.

This prevents random gestures from moving the car immediately. You first make a circle gesture to arm the bridge, then use movement gestures. Holding still or showing `STOP_IDLE` disarms the system and sends `STOP`.

The UDP payload sent from Windows looks like this:

```json

{

"cmd": "FORWARD",

"gesture": "PUSH",

"confidence": 0.84,

"hand": "Right",

"ts": 1780000000.0

}

```

The recognizer sends a heartbeat every `0.2s` with the current latched command, so the ROS side keeps receiving the command while the gesture is active.

## ROS 2 Package Cleanup

The first SolidWorks export had a package/folder name like:

```text

Car_Assem_1st.SLDASM

```

That name is awkward for ROS packages because it contains uppercase letters and a dot. The mesh paths originally used:

```text

package://Car_Assem_1st.SLDASM/meshes/...

```

For ROS 2, the package was converted to a proper description package name:

```text

car_assem_1st_description

```

Then the active second model package became:

```text

car_assembly_2nd

```

The mesh paths now use a valid package name:

```text

package://car_assembly_2nd/meshes/base_link.STL

package://car_assembly_2nd/meshes/wheel_front_left.STL

package://car_assembly_2nd/meshes/wheel_front_right.STL

package://car_assembly_2nd/meshes/wheel_rear_left.STL

package://car_assembly_2nd/meshes/wheel_rear_right.STL

```

The ROS 2 package has:

```text

package.xml

CMakeLists.txt

urdf/

meshes/

textures/

launch/

config/

scripts/

worlds/

env-hooks/

```

## URDF And TF Fixes

The SolidWorks URDF caused TF errors like:

```text

TF NAN_INPUT

TF DENORMALIZED_QUATERNION

```

These errors usually mean `robot_state_publisher` tried to publish a transform with invalid numeric values. The common causes are:

- Invalid `origin xyz`.

- Invalid `origin rpy`.

- Empty numeric fields.

- `nan`, `-nan`, or `inf`.

- A continuous or revolute joint with an invalid zero axis like `axis xyz="0 0 0"`.

- Link and joint name collisions.

The wheel area was inspected carefully. The important wheel links were:

```text

wheel_front_left

wheel_front_right

wheel_rear_left

wheel_rear_right

```

The URDF was fixed so the wheel spin joints are valid continuous joints with a non-zero local Z axis:

```xml

<joint name="wheel_front_left_spin_joint" type="continuous">

<parent link="motor_wheel_front_left" />

<child link="wheel_front_left" />

<axis xyz="0 0 1" />

</joint>

```

The same pattern was applied to:

```text

wheel_front_right_spin_joint

wheel_rear_left_spin_joint

wheel_rear_right_spin_joint

```

The motor mount joints are fixed joints:

```text

motor_wheel_front_left_mount_joint

motor_wheel_front_right_mount_joint

motor_wheel_rear_left_mount_joint

motor_wheel_rear_right_mount_joint

```

The joint names were changed so they do not collide with link names. For example:

```text

link:  wheel_front_left

joint: wheel_front_left_spin_joint

```

This matters because ROS, TF, Gazebo, and visualization tools become confusing or unstable when links and joints reuse the same names.

## RViz Visibility Fixes

RViz needed:

- A valid `/robot_description`.

- A valid TF tree.

- Mesh paths that resolve through ROS package lookup.

The expected RViz setup is:

```text

Fixed Frame: base_link

RobotModel Description Topic: /robot_description

```

The useful checks are:

```bash

cd ~/HandGestureDrone_ws

source /opt/ros/foxy/setup.bash

source install/setup.bash

check_urdf ~/HandGestureDrone_ws/src/car_assembly_2nd/urdf/car_assembly_2nd.urdf

ros2 pkg prefix car_assembly_2nd

```

## Gazebo Mesh Loading Fix

Gazebo was able to see that a model existed, but it could not load the visual mesh files.

The key concept is the installed package `share` directory. After `colcon build`, ROS installs package resources under:

```text

~/HandGestureDrone_ws/install/car_assembly_2nd/share/car_assembly_2nd

```

That installed `share` folder contains the package's URDF, meshes, textures, worlds, launch files, and other runtime assets.

When Gazebo sees paths like:

```text

model://...

package://...

```

it needs search paths that tell it where those package resources live.

An environment hook was added:

```text

/home/ros_joe/HandGestureDrone_ws/src/car_assembly_2nd/env-hooks/gazebo_model_path.dsv

```

It contains:

```text

prepend-non-duplicate;GAZEBO_MODEL_PATH;share

prepend-non-duplicate;GAZEBO_MODEL_PATH;/usr/share/gazebo-11/models

prepend-non-duplicate;GAZEBO_RESOURCE_PATH;/usr/share/gazebo-11

```

The package `CMakeLists.txt` registers the hook:

```cmake

ament_environment_hooks(

env-hooks/gazebo_model_path.dsv

)

```

After this, when you run:

```bash

source ~/HandGestureDrone_ws/install/setup.bash

```

the Gazebo search path is updated automatically.

## Why We Added A Custom Gazebo World

The ROS gesture driver controls the model through these Gazebo services:

```text

/get_entity_state

/set_entity_state

```

Those services require the Gazebo ROS state plugin:

```xml

<plugin name="gazebo_ros_state" filename="libgazebo_ros_state.so">

<ros>

<namespace>/</namespace>

</ros>

</plugin>

```

So we created:

```text

/home/ros_joe/HandGestureDrone_ws/src/car_assembly_2nd/worlds/gesture_empty.world

```

That world includes:

- `libgazebo_ros_state.so`

- `ground_plane`

- `sun`

Without this world/plugin, the driver can run but cannot control the model because `/get_entity_state` and `/set_entity_state` may not exist.

## ROS Gesture Gazebo Driver

The driver is:

```text

/home/ros_joe/HandGestureDrone_ws/src/car_assembly_2nd/scripts/gesture_gazebo_driver

```

It is installed as a ROS executable, so it can run with:

```bash

ros2 run car_assembly_2nd gesture_gazebo_driver

```

The driver:

- Listens for UDP JSON on `0.0.0.0:4210`.

- Stores the latest valid command.

- Times out to `STOP` if no UDP packet arrives for more than `1.0s`.

- Calls `/get_entity_state` at 20 Hz.

- Preserves the current pose.

- Computes a twist command.

- Calls `/set_entity_state`.

The command speeds are:

```text

FORWARD:  +0.35 m/s

BACKWARD: -0.25 m/s

LEFT:     +1.8 rad/s

RIGHT:    -1.8 rad/s

STOP:      0

```

Forward and backward are relative to the car's current yaw, not fixed world X. That means if the car rotates, forward still means "move in the direction the car is facing."

The relevant runtime dependencies were added to `package.xml`:

```xml

<exec_depend>rclpy</exec_depend>

<exec_depend>gazebo_ros</exec_depend>

<exec_depend>gazebo_msgs</exec_depend>

<exec_depend>geometry_msgs</exec_depend>

```

The script is installed through `CMakeLists.txt`:

```cmake

install(

PROGRAMS scripts/gesture_gazebo_driver

DESTINATION lib/${PROJECT_NAME}

)

```

## Build Commands

From WSL Ubuntu-20.04 as `ros_joe`:

```bash

cd ~/HandGestureDrone_ws

source /opt/ros/foxy/setup.bash

colcon build --packages-select car_assembly_2nd

source install/setup.bash

```

Verify that ROS can find the executable:

```bash

ros2 pkg executables car_assembly_2nd

```

Expected:

```text

car_assembly_2nd gesture_gazebo_driver

```

## Final Run Commands

Use separate terminals.

### Terminal 1: Start Gazebo

```bash

source /opt/ros/foxy/setup.bash

source ~/HandGestureDrone_ws/install/setup.bash

export GAZEBO_MODEL_DATABASE_URI=""

export LIBGL_ALWAYS_SOFTWARE=1

export QT_X11_NO_MITSHM=1

gazebo --verbose \

~/HandGestureDrone_ws/install/car_assembly_2nd/share/car_assembly_2nd/worlds/gesture_empty.world \

-s libgazebo_ros_init.so \

-s libgazebo_ros_factory.so

```

### Terminal 2: Spawn The Car

```bash

source /opt/ros/foxy/setup.bash

source ~/HandGestureDrone_ws/install/setup.bash

ros2 run gazebo_ros spawn_entity.py \

-entity car_assembly_2nd \

-file ~/HandGestureDrone_ws/src/car_assembly_2nd/urdf/car_assembly_2nd.urdf

```

### Terminal 3: Start The Gesture Driver

```bash

source /opt/ros/foxy/setup.bash

source ~/HandGestureDrone_ws/install/setup.bash

ros2 run car_assembly_2nd gesture_gazebo_driver

```

Expected driver logs:

```text

Listening for UDP commands on 0.0.0.0:4210

Controlling Gazebo model 'car_assembly_2nd'

Gazebo entity state services are ready

Active command: STOP source=timeout

```

### Terminal 4: Start The Windows Recognizer

In PowerShell:

```powershell

cd "C:\Users\Yoav\Desktop\Personal Projects\hand-gestured-car"

.\.venv\Scripts\activate

Remove-Item Env:GESTURE_BRIDGE_HOST -ErrorAction SilentlyContinue

python .\src\live_recognize.py

```

Expected recognizer log:

```text

[Live Recognize] Running.

Controls: Q quit | A toggle armed | circles arm | STOP disarms

[Gesture Bridge] Sending UDP commands to 172.22.129.75:4210 (auto WSL Ubuntu-20.04/ros_joe eth0)

```

The IP can change after restarting WSL, but the script now auto-detects it.

## The Routing Problem We Found

The confusing part was that the recognizer appeared to work:

```text

[ARMED] via CIRCLE_CW

[EVENT] PUSH -> FORWARD

[EVENT] PULL -> BACKWARD

```

But the car did not move.

The first bad sign was:

```text

[Gesture Bridge] Sending UDP commands to 127.0.0.1:4210

```

On Windows, `127.0.0.1` means Windows itself. It does not necessarily reach the UDP socket inside WSL.

Then we changed the recognizer to auto-detect WSL, but it initially detected the wrong WSL instance:

```text

172.22.128.1

```

The real ROS/Gazebo system was running in:

```text

Ubuntu-20.04

user: ros_joe

```

But plain `wsl` was entering a different default distro/user:

```text

Ubuntu-18.04

home: /home/yoavmatalon

```

So the script was detecting an IP from the wrong Linux environment.

The correct check was:

```powershell

wsl -d Ubuntu-20.04 -u ros_joe hostname -I

```

That returned:

```text

172.22.129.75

```

The recognizer was patched so `resolve_bridge_host()` explicitly checks:

```text

distro: Ubuntu-20.04

user:   ros_joe

```

It now prints:

```text

[Gesture Bridge] Sending UDP commands to 172.22.129.75:4210 (auto WSL Ubuntu-20.04/ros_joe eth0)

```

## Manual UDP Test That Proved It Worked

We manually sent UDP packets from Windows to the corrected WSL IP:

```powershell

@'

import json

import socket

import time

addr = ("172.22.129.75", 4210)

sock = socket.socket(socket.AF_INET, socket.SOCK_DGRAM)

for cmd, duration in [("FORWARD", 1.5), ("STOP", 0.3)]:

msg = json.dumps({

"cmd": cmd,

"gesture": "manual",

"confidence": 1.0,

"hand": "test",

"ts": time.time()

}).encode("utf-8")

end = time.time() + duration

while time.time() < end:

sock.sendto(msg, addr)

time.sleep(0.05)

print("sent manual FORWARD then STOP to", addr)

'@ | python -

```

Before the test, Gazebo reported the model near:

```text

x=-0.1428, y=-0.0804

```

After the test, Gazebo reported:

```text

x=0.0450, y=-0.0330

```

That proved:

- Gazebo had the model.

- `/get_entity_state` worked.

- `/set_entity_state` worked.

- The driver was receiving UDP.

- The car moved when packets were sent to the correct IP.

## Useful Debug Commands

Check the correct WSL IP:

```powershell

wsl -d Ubuntu-20.04 -u ros_joe hostname -I

```

Check which WSL distro is default:

```powershell

wsl -l -v

```

Check whether Gazebo and the driver are running:

```powershell

wsl -d Ubuntu-20.04 -u ros_joe sh -lc "ps -ef | grep -E 'gesture_gazebo_driver|gazebo|gzserver|gzclient|spawn_entity' | grep -v grep"

```

Check whether the UDP port is already in use in the correct WSL distro:

```powershell

wsl -d Ubuntu-20.04 -u ros_joe python3 - <<'PY'

import socket

s = socket.socket(socket.AF_INET, socket.SOCK_DGRAM)

try:

s.bind(("0.0.0.0", 4210))

print("udp_4210_free")

except OSError as e:

print("udp_4210_in_use", e)

PY

```

If the driver is running, expected output is:

```text

udp_4210_in_use [Errno 98] Address already in use

```

Check Gazebo state services:

```bash

source /opt/ros/foxy/setup.bash

source ~/HandGestureDrone_ws/install/setup.bash

ros2 service list | grep -E '/get_entity_state|/set_entity_state'

```

Expected:

```text

/get_entity_state

/set_entity_state

```

Check whether Gazebo can see the car model:

```bash

source /opt/ros/foxy/setup.bash

source ~/HandGestureDrone_ws/install/setup.bash

ros2 service call /get_entity_state gazebo_msgs/srv/GetEntityState "{name: car_assembly_2nd, reference_frame: world}"

```

Expected:

```text

success=True

```

## Manual Override For The Bridge IP

If auto-detection fails, set the host manually in PowerShell:

```powershell

$env:GESTURE_BRIDGE_HOST="172.22.129.75"

python .\src\live_recognize.py

```

To go back to auto-detect:

```powershell

Remove-Item Env:GESTURE_BRIDGE_HOST -ErrorAction SilentlyContinue

python .\src\live_recognize.py

```

The script also supports these optional environment variables:

```powershell

$env:GESTURE_WSL_DISTRO="Ubuntu-20.04"

$env:GESTURE_WSL_USER="ros_joe"

$env:GESTURE_BRIDGE_PORT="4210"

```

## Current Limitations

This is a V1 control loop. It prioritizes "gesture makes the Gazebo model move" over physical realism.

Current limitations:

- The car is moved with direct Gazebo entity velocity, not wheel physics.

- Wheels do not yet drive the car through contact/friction.

- There is no differential drive plugin yet.

- There is no ROS 2 control hardware interface yet.

- If the hand is held still, `STOP_IDLE` can disarm the bridge.

- The recognizer process must be restarted after changing environment variables or code.

## Next Improvements

Good next steps:

- Add a small on-screen status indicator for the selected UDP host and armed state.

- Add a console print when each UDP heartbeat is sent, only while debugging.

- Add a ROS topic output like `/gesture_cmd` for visibility in `ros2 topic echo`.

- Convert direct Gazebo control to a differential drive setup.

- Connect wheel joints to a Gazebo plugin or ROS 2 control.

- Add speed parameters to the driver launch file.

- Add a proper launch file that starts Gazebo, spawns the car, and starts the driver together.

## Short Version

The project works because these pieces are now aligned:

1. The URDF package is a valid ROS 2 package.

2. Mesh paths resolve through `package://car_assembly_2nd/...`.

3. Gazebo can find installed model assets through the environment hook.

4. The custom Gazebo world loads `libgazebo_ros_state.so`.

5. The ROS driver listens on UDP port `4210`.

6. The Windows recognizer sends commands to the correct WSL Ubuntu-20.04 IP.

7. The driver converts gesture commands into Gazebo model velocity.

The final key fix was not the gesture model. It was routing: the recognizer was sending commands to the wrong address because Windows had multiple WSL environments. Once it explicitly targeted `Ubuntu-20.04` as `ros_joe`, the UDP packets reached the running ROS driver and the car moved.